SLOVENYA LİMANLARINDA TÜRK BAYRAKLI GEMİLERDE 2022-2026 YILLARI ARASINDA TESPİT EDİLEN EKSİKLİKLER

(Tutulma maddeleri Gri Renkli olarak verilmiştir)

| DENETİM LİMANI | DENETİM TARİHİ | NO | REFERANS | EKSİKLİK |
| --- | --- | --- | --- | --- |
| KOPER | 28.01.2023 | 1 | MARPOL ANNEX I-R14 | BWS pump-pressure gauge malfunctioning. |
|  | 06.02.2024 | 1 | STCW CODE PART A CH.II | Minimum safe manning requirement is for 3 deck ratings holding certificates- STCW II/4 or II/5. Two seamen are performing bridge watch with no certification STCW II/4 or II/5. |
|  |  | 2 | MLC TITLE 4 | Acetylene, oxygene and propane gas cylinders stored together, not lashed properly. |
|  |  | 3 | SOLAS CH. II-1 | Emergency generator exhaust pipe has exhaust smoke leakage in the emergency generator room, cooler blower not ready for use, damaged cooler radiator, no evidence fort he last fuel oil filters change. |
|  |  | 4 | SOLAS CH.II-2 | Main engine fuel oil leakage alarm collecting tank – damaged, alarm sensor probe not fixed. |
|  |  | 5 | MARPOL AN. I | OWS – malfunction of the pneumatic valve fordiverting of oily water to the bilge tank. |
|  |  | 6 | MARPOL AN. IV | Sewage treatment plant – malfunction of the air supply pump. |
|  |  | 7 | SOLAS CH.II-1 | All three auxiliary engines – missing some safety radiator caps fort he coolers, some caps not tight in position. |
|  |  | 8 | LL / R. 20 | Fuel oil tank vents (Overflow fuel tank and Thermal oil drain) on main deck, STBD side – heavilly corroded. |
|  |  | 9 | MLC TITLE4 | Both anchors swivel shackles with joint shackles – material wastage. |
|  |  | 10 | ISM CODE PART A/1 | Corrective action taken on the ISM system by the Company is required within 3 months. Deficiencies marked ISM are objective evidence of a failure, or lack of effectiveness, of the implementation of the ISM Code. The ship will be eligible for reinspection after 3 months from the final date of the reporrt. |
|  | 09.05.2024 | 1 | SOLAS CH.V R. 19 | S-band radar- TX time is 25038 hours. X- band radar- TX time is 28514 hours. Manufacturer reccommendation for replacement of the magnetron is after 5000 TX time. |
|  |  | 2 | SOLAS CH.III | Lifeboats ( Ps and STBD ) – steel foundation support for boat hooks ( above propeller ) - heavilly corroded, huge vastage of material. |
|  |  | 3 | MLC 2006 TITLE 4 | Emergency generator - missing V-belts protection. |
|  |  | 4 | SOLAS CH.II-2 | Engine room vent door damper ( stbd ) - malfunction of the door closing handle. |

| KOPER | 30.11.2024 | 1 | SOLAS CH.II-2 | Fire line on the main deck - branch lines between cargo holds - heavily corroded on several spots. |
| --- | --- | --- | --- | --- |
|  |  | 2 | SOLAS CH.II-2 | Machinery space vent damper (No.3 ) - malfunction of the actuator valve when closing manually directly by the damper. Remote closing from the fire station is in good order. |
|  |  | 3 | MLC 2006 TITLE-4 | Aft mooring winches ( portside and middle one ) - hydraulic leakages. |
|  |  | 4 | MLC 2006 TITLE-4 | Stairway from main deck to the forecastle (PS) - heavily corroded and corroded through on several spots. |
|  |  | 5 | MARPOL ANNEX IV | Sewage treatment plant - air supply pump out of order. |