ESTONYA LİMANLARINDA TÜRK BAYRAKLI GEMİLERDE 2022-2026 YILLARI ARASINDA TESPİT EDİLEN EKSİKLİKLER

(Tutulma maddeleri Gri Renkli olarak verilmiştir)

| DENETİM LİMANI | DENETİM TARİHİ | NO | REFERANS | EKSİKLİK |
| --- | --- | --- | --- | --- |
| MUUGA | 09.06.2024 | 1 | SOLAS CH.I | Ship is described as 'other cargo ship' in CSSCC, CSCEC, SMC and DoC. In Certificate of Registry, MSMD and in Class certificate it is as 'Bulk Carrier. Vessel has double skinned holds and is constructed before 01.07.2006. Flag State to Investigate and if necessary to re-issue flag certificates. |
|  |  | 2 | SOLAS CH.III | Lights of the lifejackets are hanging by the rope and not providing visibility over as great a segment of the upper hemisphere as is practicable when attached to a lifejacket |
|  |  | 3 | SOLAS CH.II | Several emergency lights on deck are not working. Overboard light at the port side embarkation station is not working. |
|  |  | 4 | SOLAS CH.II | Found one fire hose leaking. |
|  |  | 5 | ISM CODE-SOLAS CH.IX REG.3 | Corrective action taken on the ISM system by the Company is required within 3 months. Deficiencys marked ISM are objective evidence of a failure, or lack of effectiveness, of the implementation of the ISM Code. The ship will be eligible for reinspection after 3 months from the final date of the report. |